PART-A

- AC and DC charging differences :

| Feature | AC Charging | DC Charging |
| --- | --- | --- |
| Conversion Location | Inside the vehicle (On-board) | Outside the vehicle (Station) |
| Charging Speed | Slow to Moderate | Fast to Ultra-Fast |
| Power Output | Typically 7 kW – 22 kW | 50 kW – 350+ kW |
| Best For | Daily/Overnight use | Long trips/Quick top-ups |
| Cost | Low | High |

- Where it used:

- Highway Rest Stops: Strategically located every 50–100 km to enable long-distance interstate travel and road trips.

- Range Anxiety Mitigation: Provides drivers with the confidence to travel beyond their vehicle's single-charge limit.

- Transit Refueling: Allows travelers to recharge to 80% during a brief meal or coffee break, typically in 20–45 minutes.

- High-Power Output: These stations offer $150 \text{ kW}$ to $350 \text{ kW}$ to maximize the energy transfer rate for modern EVs.

- Commercial Fleet Operations: Essential for electric taxis and ride-sharing vehicles that require maximum uptime during shifts.

- Last-Mile Delivery: Enables delivery vans to perform "opportunity charging" between routes to complete a full day's work.

- Public Transit: Used at bus depots for rapid turnaround of electric buses between scheduled city runs.

- Dedicated Urban Hubs: Serve as a modern, high-speed alternative to traditional petrol stations in high-density city centers.

- "Garage-Orphan" Support: Provides a vital solution for EV owners in apartments who lack access to private overnight AC charging.

- Emergency Services: Ensures electric police cruisers or ambulances can return to a high state of charge immediately after a call.

- Industrial Logistics: Keeps electric heavy-duty trucks moving by providing rapid power at freight terminals and ports.

- Retail Integration: Found at large shopping centers to allow customers to gain significant range while running errands.

- Grid Balancing: Modern hubs often use integrated battery storage to manage high electricity demand without straining the grid.

- Predictable Downtime: Specifically designed for environments where vehicles are stationary for less than an hour.

- Infrastructure Growth: Represents the "backbone" of the global transition toward sustainable, long-range electric mobility.

- Advantages and Limitations:

- Rapid Recovery: Can charge an EV from 10% to 80% in just 20 to 45 minutes, significantly reducing wait times.

- Travel Essential: Provides the necessary convenience for long-distance travel where waiting hours for an AC charge is not feasible.

- High Infrastructure Investment: Extremely costly to install due to the requirement for high-capacity transformers and specialized cooling systems.

- Premium Usage Fees: Much more expensive for the end-user per kWh compared to residential or public AC charging rates.

- Direct Battery Feed: Bypasses the vehicle's internal on-board charger to send high-voltage DC power directly to the battery cells.

- Thermal Stress: Rapid energy transfer generates significant heat, which requires sophisticated thermal management systems within the vehicle.

- The 80% Threshold: Charging speeds typically throttle down after 80% to protect the battery chemistry from degradation and overheating.

- Grid Demand: Requires a heavy-duty connection to the electrical grid, often needing its own dedicated substation or high-voltage line.

- Equipment Footprint: The charging units are much larger than AC pedestals because they house massive power conversion electronics.

- Site Preparation Costs: Installation often involves extensive civil engineering, including trenching and upgrading local utility capacity.

- Battery Longevity Impact: Frequent and exclusive use of ultra-fast DC charging can accelerate the long-term wear of the battery pack.

- Vehicle Compatibility: The maximum charging speed is often limited by the vehicle’s maximum intake capacity (e.g., a 50 kW car cannot use the full 350 kW of a charger).

- Active Cooling Cables: Many high-power DC chargers use liquid-cooled cables to manage the heat generated by high amperage.

- Strategic Positioning: Most effective when placed in "high-turnover" areas where drivers spend less than an hour, such as highway plazas.

- Operational Complexity: Maintenance costs are higher for operators due to the complex electronics and moving parts like cooling fans and heavy cables.

PART-B

Charging Standards:

| Standard | Power Type | Typical Power Range | Region / Primary Use |
| --- | --- | --- | --- |
| Type 1 (SAE J1772) | AC (Single Phase) | Up to 7.4 kW | North America, Japan |
| Type 2 (Mennekes) | AC (1-Phase / 3-Phase) | 7.4 kW – 43 kW | Europe, India (Standard) |
| CCS (Combined Charging System) | DC Fast Charging | 50 kW – 350+ kW | Global (CCS1 in US, CCS2 in Europe/India) |
| CHAdeMO | DC Fast Charging | 50 kW – 400 kW | Japan (Global legacy) |
| Bharat AC-001 | AC (3-Phase) | 10 kW (3.3 kW x 3) | India (Low-cost) |
| Bharat DC-001 | DC Fast Charging | 15 kW / 72V - 200V | India (Public Fleets) |

PART-C

| Scenario | Battery Capacity (kWh) | Charging Power (kW) | Calculation | Total Charging Time |
| --- | --- | --- | --- | --- |
| 1 | 30 kWh | 3 kW | 330​ | 10 hours |
| 2 | 50 kWh | 25 kW | 2550​ | 2 hours |
| 3 | 75 kWh | 50 kW | 5075​ | 1.5 hours (90 mins) |

PART-D

| Battery Size | Efficiency | Theoretical Range | Best Use Case |
| --- | --- | --- | --- |
| 50 kWh | 5 km/kWh | 250 km | Daily city commuting |
| 60 kWh | 6.5 km/kWh | 390 km | Mixed city/highway use |
| 75 kWh | 7 km/kWh | 525 km | Long-distance touring |

Real-world factors:

Driving Style: Frequent aggressive acceleration and heavy "braking" (without utilizing regenerative braking effectively) consume more energy than smooth, gradual inputs.

Speed: EVs are most efficient in city traffic. At high highway speeds, aerodynamic drag increases exponentially, significantly lowering the km/kWh efficiency.

Weather: Batteries have an optimal operating temperature. In extreme cold, the battery’s chemical reactions slow down, and in extreme heat, energy is diverted to cool the battery pack.

AC Usage: Unlike petrol cars that use waste engine heat for the cabin, EVs must use battery power for both heating and cooling (HVAC). Heavy air conditioning or heater use can reduce range by 10–20%.

Load: Every extra kilogram requires more torque to move. Carrying heavy cargo or a full car of passengers increases the rolling resistance and energy consumption.

PART-E

| Parameter | Specification |
| --- | --- |
| Battery Capacity | 45 kWh (Lithium-ion LFP) |
| Ideal Range (MIDC) | 489 km |
| Real-World Range | Approx. 320–350 km (Condition dependent) |
| Charging Standards | CCS-II (Combined Charging System Type 2) |
| AC Charging Time | ≈6.5 hours (using 7.2 kW Wall Box, 10% to 100%) |
| DC Charging Time | ≈40 minutes (using 60 kW Fast Charger, 10% to 100%) |

Analysis: Ideal vs. Real-World Range

There is a significant delta between the "Ideal" (MIDC) range and what a driver experiences on the road.

- The "80% Rule": While the ideal range assumes 100% to 0% discharge, real-world users typically operate between 10 – 80 to preserve battery health, effectively reducing usable range by 30%.

- Efficiency Loss: The MIDC cycle is tested in controlled lab environments. In reality, factors like regenerative braking levels (Nexon has 4 levels) significantly impact efficiency. In heavy bumper-to-bumper traffic, high regen can actually extend range, whereas constant highway speeds 100 km/h cause it to drop sharply due to aerodynamic drag.

3. Charging Infrastructure Challenges (India Focus)

As of 2026, several bottlenecks remain for widespread adoption:

- Grid Constraints: A single DC fast charger requires a sanctioned load of 300 kW. Many existing commercial buildings and highway stops require expensive transformer upgrades to support this.

- Low Uptime: While the number of chargers has increased 5x since 2022, many suffer from maintenance issues, software glitches, or payment gateway failures, leading to "charger anxiety."

- Standardization: While CCS-II is the primary standard for four-wheelers, the coexistence of older standards and proprietary networks (like Tesla's Supercharger vs. open networks) can confuse new users.

4. Efficiency Comparison

Compared to its peers like the Tesla Model 3 or Hyundai Kona, the Nexon EV is highly optimized for tropical climates.

- Nexon EV: Uses LFP (Lithium Iron Phosphate) chemistry, which is heavier and less energy-dense than Tesla’s NMC cells but significantly more stable in high ambient temperatures (common in India) and has a longer cycle life.

- Consumption: At an average of 7 km/kW, it is one of the most efficient compact SUVs globally. For comparison, a Tesla Model 3 RWD achieves roughly 7.5 km/kWh due to its superior aerodynamics, despite being a larger car.